GWO-FOPSS / GWO-PSS

MATLAB / Simscape Electrical Simulation Package

MATLAB R2025a  |  Simscape Electrical Required

GWO-FOPSS: A Robust Fractional-Order Power System Stabilizer Controller Optimized via Grey Wolf Optimizer

Authors: Amal Ibrahim Nasser 1, 2, 3

Contact: amalalshemmiri@uomustansiriyah.edu.iq;

Copyright (c) 2026, All rights reserved.

# 1. Overview

This package implements and compares three Power System Stabilizer (PSS) designs for a Single-Machine Infinite-Bus (SMIB) system with a salient-pole hydroelectric generator (32 poles):

- NOPSS— the system without pss

- CPSS — Conventional PSS with fixed integer-order lead-lag structure

- GWO-PSS — Integer-order PSS with parameters tuned by the Grey Wolf Optimizer

- GWO-FOPSS — Fractional-Order PSS with parameters tuned by the Grey Wolf Optimizer (proposed method)

The optimizer minimizes the Integral of Time-weighted Absolute Error (ITAE) of the rotor speed deviation following a transient disturbance, ensuring robust damping of low-frequency oscillations.

# 2. Repository Structure

| File / Folder | Description |
| --- | --- |
| System_Model.slx | Simscape Electrical nonlinear SMIB model with GWO-FOPSS controller |
| System_Model2.slx | Simscape Electrical nonlinear SMIB model with GWO-PSS (integer-order) |
| nolpss.slx | Simscape Electrical model without PSS |
| conventinalpss.slx | Simscape Electrical model with conventional CPSS (fixed parameters) |
| GWOpss.slx | Simscape Electrical model with GWOPSS (optimization parameters) |
| GWOFOPSS.slx | Simscape Electrical model with GWOFOPSS (optimization parameters) |
| GWOCode.m | GWO Optimization for PSS Controller (4 Parameters) Optimized Variables: Kp, T1, T2, Tw |
| GWO_FOPSSCode.m | GWO-FOPSS Optimization (5 Parameters) Optimized Variables: K, Tw, T1, T2, Alpha |
| oustaloup.m | OUSTALOUP Fractional Order Approximation |
| FOPSS_Final_Results.mat | Pre-computed GWO optimization results and simulation data |
| README.md | This documentation file |

# 3. Quick Start

## Step 1 — Open MATLAB and set the working directory

cd('path/to/gwo')

## Step 2 — Load pre-computed results (optional)

To skip optimization and use the published results directly:

load('FOPSS_Final_Results.mat')

## Step 3 — Run a simulation model

Open the desired Simulink model from MATLAB:

open_system('System_Model.slx')      % GWO-FOPSS (proposed)

open_system('System_Model2.slx')     % GWO-PSS   (integer-order)

open_system('conventinalpss.slx')    % CPSS      (conventional)

Then press Run in Simulink, or simulate from the command line:

sim('System_Model')

## Step 4 — Run GWO optimization (re-tune parameters)

If you wish to re-run the optimization from scratch, execute the GWO script embedded in System_Model.slx via the Model Workspace, or call the optimizer directly if exported as a standalone script. The optimizer will tune the parameter vector:

θ = [ KPSS ,  Tw ,  T1 ,  T2 ,  α ]

The fractional order α is active only for the FOPSS model; it is fixed at α = 1 in the integer-order (GWO-PSS) case.